Detailed Manual Guide

Here is the complete, step-by-step assembly guide for building the thermo-mechanical material sorting prototype, tailored for a first-time hardware setup.

Phase 1: Mechanical & Structural Assembly

The goal of this phase is to align and rigidly couple the 42 stepper motor, linear slide module, load cell sensor, and NV6 heater block into a single vertical assembly.

┌────────────────────────┐

│   NEMA 17 Stepper      │ (Top)

└───────────┬────────────┘

│

┌──────────────┴──────────────┐

│   T8 Lead Screw Slide       │ (Vertical Carriage)

└──────────────┬──────────────┘

│

┌───────────┴────────────┐

│  YZC133 Load Cell      │ <-- Mounted as a cantilever ("I-beam" load direction)

└───────────┬────────────┘

│

┌───────────┴────────────┐

│  NV6 Heater Block      │

└───────────┬────────────┘

│

┌───────────┴────────────┐

│ Volcano Stainless Probe│

└────────────────────────┘

- Mount Motor to Slide Module

  - Insert the shaft of the NEMA 17 stepper motor into the flexible coupling at the top of the linear slide.

  - Secure the motor body to the bracket using screws, then tighten the grub screws on the side of the flexible coupling.

- Attach Load Cell Sensor (YZC133)

  - Locate the sensor's load orientation arrow indicator.

  - Fasten one end of the load cell bar to the moving carriage block of the linear slide using M4 screws.

  - Crucial: Ensure the middle section of the bar remains unconstrained (cantilever setup). Bolt the metal bracket of the NV6 Heater Block exclusively to the opposite free end of the sensor.

- Assemble the Heating Probe

  - Thread the 0.4mm Volcano Stainless Steel Nozzle into the bottom threaded hole of the NV6 aluminum heater block and tighten it securely.

  - Insert the NTC100K thermistor and the 12V ceramic cartridge heater into their respective side bores, then tighten the retaining screws to keep them in place.

Phase 2: Wiring & Electrical Setup

Main power is supplied via the 12V 5A power supply, which powers the heating element, motor driver, and is stepped down to 5V through the MP1584 Buck Converter for the ESP32 and logic modules.

1. Power Distribution & Step-Down

- 12V Power Supply Output $\rightarrow$ MP1584 Buck Converter Input (IN+ / IN-).

- MP1584 Output (OUT+ / OUT-, set/fixed at 5V) $\rightarrow$ ESP32 Expansion Board (5V and GND pins / DC jack).

- 12V Power Supply Output $\rightarrow$ IRF520 MOS Module Power IN (+/-) AND A4988 Driver Expansion Board VMOT/GND.

2. Control & Sensor Connections (ESP32 Node)

| Module | Pin Name | Connected to ESP32 Board / Node | Notes |
| --- | --- | --- | --- |
| 0.91/0.96" OLED Display | VCC / GND | 5V (or 3.3V) / GND | Logic power |
|  | SCL / SDA | GPIO 22 / GPIO 21 | I2C Default Pins |
| HX711 Amplifier | VCC / GND | 5V / GND | Logic power |
|  | DT / SCK | GPIO 16 / GPIO 17 | Data & Clock Pins |
|  | E+ E- A+ A- | YZC133 Wire Harness | Red: E+, Black: E-, White: A+, Green: A- |
| A4988 Driver Board | VCC / GND | 5V / GND | Logic power |
|  | STEP / DIR | GPIO 18 / GPIO 19 | Motion control pins |
|  | 1A 1B 2A 2B | Stepper Motor 4-pin Harness | Motor phase coils |
| IRF520 MOSFET Module | SIG / GND | GPIO 23 / GND | PWM output signal pin |
|  | Power OUT | NV6 Ceramic Heater Cartridge | Non-polar (either wire) |
| NTC100K Thermistor | Terminal Line 1 | 3.3V | Voltage divider circuit |
|  | Terminal Line 2 | GPIO 34 (ADC1_CH6) | Note: Pull down this node to GND using the 4.7kΩ resistor. |

Phase 3: Firmware & Calibration Workflow

Step 1: Flashing Base Testing Code (Arduino IDE)

- Install Arduino IDE with the ESP32 Board Support Package.

- Install the required libraries via Library Manager:

  - Adafruit_SSD1306 (OLED Display Control)

  - HX711 (Load Cell ADC Interface)

  - AccelStepper or Stepper (Motor Step/Dir Timing)

- Connect the ESP32 to your PC via data-capable USB cable, selecting ESP32 Dev Module and the correct COM port.

Step 2: Component Calibration

- Temperature & PID Verification:

  - Flash a basic analog-read code snippet to verify thermistor ambient temperature readings (~25°C).

  - Pulse the MOSFET via GPIO 23 high-state to verify heater response and reach the targeted glass transition temperature (~70°C).

- Load Cell Tare Calibration:

  - Run the HX711 scale calibration sketch with the probe suspended freely in air to set the zero offset (tare).

- Downward Penetration Test:

  - Send stepper motor downward step commands (e.g., 100 steps/sec).

  - Monitor real-time force data via the HX711. As the probe makes contact with a test piece and meets resistance, verify that measured weight/force increases rapidly.

Practical Assembly Tips

- Terminal Screw Tightness: High-current lines (12V input and MOSFET output) must be clamped tightly in screw terminals to prevent thermal buildup or loose connections.

- A4988 Potentiometer Adjustment: Use a small screwdriver to turn the tiny potentiometer on the A4988 module. If the motor gets excessively hot, rotate slightly counter-clockwise to reduce Vref. If it clicks or skips steps during downward pressing, rotate slightly clockwise to boost torque.

- Common Ground (GND): Ensure the GND pins of the ESP32, HX711, MOSFET, and 12V PSU share a common ground reference to avoid signal drift.

Preliminary Ideas for PLA Identification

不纯的PLA可能会有问题

找一找需要的设备，以及现成的产品

方案一：高性价比方案——热敏与硬度多传感器探针阵列（推荐创客首期）

工业回收常利用高价设备，而创客项目可以通过 PLA 独特的物理特性（较低的玻璃化转变温度和硬度） 搭建低成本测试台。

1. 识别原理

- 玻璃化转变温度（$T_g$）差异：PLA 的 $T_g$ 极低（约 55-65℃），在此温度下就会软化；而 ABS（约 105℃）、PETG（约 80℃）在此温度下依然保持坚硬。

- 密度差异：PLA 密度约为 $1.24\,\text{g/cm}^3$，ABS 为 $1.04\,\text{g/cm}^3$，PETG 为 $1.27\,\text{g/cm}^3$。

2. 装置结构设计

- 加料传送带：将碎料（或未剪断的线头/小块废件）单排送入检测区。

- 受控加热压针探头：

  - 在机械臂或气缸末端安装一个温控微型加热头（恒温设定在 70℃），并集成压力传感器（如 HX711 + 应变片）。

  - 探头按压废料表面：PLA 在 70℃ 下迅速软化，压针深入（位移大、抵触力骤降）；ABS/PETG 保持坚硬（位移小、抵触力高）。

- 分选执行机构：单片机（ESP32 / Arduino）根据位移与压力变化判断是否为 PLA，并通过舵机拨板或气压喷嘴将 PLA 剔除至回收桶。

方案二：进阶智能化方案——近红外光谱识别（NIR）

工业界最主流的塑料分选技术是 近红外光谱法（NIR）。PLA 含有独特的酯键（C=O 和 C-O），在 1100–1700 nm 波段有非常显著的吸收峰。

1. 核心硬件选型

- 微型光谱仪模块：如 AS7263（6通道近红外传感器）或 TI NIRScan Nano 开源模块。

- 光源：高功率卤素灯或宽谱红外发光二极管（用于提供稳定的红外反射光源）。

- 主控：树莓派（Raspberry Pi 4 / 5）或 Jetson Nano。

2. 工作流程与算法

[光源照射废料] ➔ [读取光谱反射率] ➔ [特征波长提取] ➔ [SVM/KNN分类模型] ➔ [输出结果: PLA / ABS / OTHER]

- 样本采集：收集常见 3D 打印废料（PLA、ABS、PETG、TPU 各 50 组样本），采集近红外波段反射数据。

- 特征匹配：PLA 在 1200nm 与 1450nm 附近的吸收特征与 ABS（苯环特征）差异极大。利用简单的机器学习算法（如 SVM 支持向量机或决策树）即可实现 95% 以上的识别率。

如何用“现成模块”快速组装一台？

你完全不需要从零画 PCB 或做复杂的机械加工，直接采购以下开源硬件模块即可拼接：

[加热与温控模块] ➔ 3D打印机加热块 (V6 Heater Block) + NTC100K Thermistor

[力学感知模块] ➔ HX711 称重传感器模块 + 压电/压力触头

[位移感知模块] ➔ 步进电机 (NEMA17) / 舵机 + 光轴编码器 (或直接用 3D 打印机 Z 轴)

[主控与算法]   ➔ ESP32 / Arduino UNO

搭建步骤：

- 加热头：直接买 3D 打印机的 V6 加热块套管（成本約 15 元），里面插上 12V/24V 加热棒和热敏电阻，将温度精准控制在 70℃。

- 探针机构：在加热块前端装一根金属钝针。把整个加热块固定在一个带有 HX711 称重传感器 的悬臂或滑轨上。

- 动作逻辑：

  - 步进电机驱动探针向下压废料，深度固定为 1mm。

  - 如果物料是 PLA：在 70℃ 下软化，探针轻松刺入，压力传感器测得的阻力值低（例如 $< 200\,\text{g}$）。

  - 如果物料是 ABS/PETG/Nylon：在 70℃ 下依然非常坚硬，阻力剧增（例如 $> 1500\,\text{g}$），甚至触发过载保护。

  - ESP32 依据 压力 / 位移 的比值（即热态表观刚度）做出逻辑判断。

3DSWAY 3D打印机配件NV6加热块铝块硅胶套件电阻偶测温兼容e3d v6-淘宝网 材质随意

Volcano不锈钢E3D火山口喷嘴黄铜喷头大孔径喷嘴头硬化钢Vyper M6-淘宝网 不锈钢 1.75/0.4mm 规格

高精度微型称重传感器https://detail.tmall.com/item.htm?abbucket=7&id=942579684297&mi_id=0000pIboo3rTn4zZ4KduKRKEV_kM8fBEsTLgvU5FQwmhCWY&ns=1&priceTId=2100c80a17853928007864974e0a2d&skuId=5938330237268&spm=a21n57.1.hoverItem.1&utparam=%7B%22aplus_abtest%22%3A%227cdd4e418bca0ed2689e43a22e160f18%22%7D&xxc=taobaoSearch 小型厨房克数秤1/2/5kg电子实验广测YZC133-tmall.com天猫 选择2kg

HX711称重模块实验套件DIY电子秤压力传感器单片机支持STM32开发-tmall.com天猫

A4988步进电机驱动器 3D打印机配件DRV8825驱动模块板 16/32细分-淘宝网 选A4988

hanpose42步进电机NEMA17可控制正反转12V高速小马达厂家支持定制-tmall.com天猫 选42电机 40高MM 轴径5MM

4pin杜邦头电机线 标配42步进马达连接XH2.54端子现货 长度可定制-tmall.com天猫 选杜邦线2.54

原厂供货0.91 0.96 1.3寸OLED显示屏模块IIC通讯SH1106/1306驱动-淘宝网 0.91寸即可

12V5A开关电源 小体积 S-60-12 12V60W-淘宝网

MP1584EN DC-DC直流降压3A电源降压可调模块24V固定转12V9V5V3V-淘宝网 选5V

IRF520驱动模块 MOS管场效应管单片机驱动模块 PWM调节-tmall.com天猫

DZQJ A4988/DRV8825步进电机驱动器模块扩展板3D打印机配件-tmall.com天猫 选A4988

金属膜电阻器1/4W元件1%色环2k 1K 4.7K 10K 100K 100欧120欧姆1M-tmall.com天猫 选4.7K

还需要的材料（已有）

面包板

3D 打印机的 Z 轴组件

ESP32

typeC数据线

DC5.5x2.1mm 转螺丝接线端子（母头）

系统架构

控制与显示：ESP32 开发板、0.96吋 OLED 屏（I2C 4针版）。

感知系统：YZC133 称重传感器（选 1kg 或 2kg 足够）、HX711 转换模块、NTC100K 热敏电阻、4.7kΩ 上拉电阻。

加热系统：NV6 加热块套件（含加热棒）、IRF520 MOS管模块、M6 不锈钢直柱探针。

动力机械：42 步进电机、A4988 驱动板 + 扩展底座、3D 打印机 Z 轴组件（含丝杆）。

电源供电：12V 5A 开关电源、MP1584EN 降压模块（调至 5V 给 ESP32 供电）。

线材与结构：面包板、杜邦线、DC 转接头。

组装流程

. 电路与信号接线指南（无需深厚电路知识）

把 ESP32 当作“大脑”，按照功能分成三个子系统连接：

A. 供电主线路（给全车供电）

- 12V 开关电源 输出两路：

  - 第一路：接 电机驱动器（如 A4988） 的 VMOT 电源口。

  - 第二路：接 LM2596 降压模块 的输入端，调整输出为 5V 后，接 ESP32 的 VIN（或 5V）及 GND 引脚。

B. 加热控制回路（控温部分）

- 测温：V6 加热块上的 NTC100K 热敏电阻（两根线），一根接 ESP32 的 GND，另一根接 ESP32 的模拟引脚（如 VP / 36），并按标准电路接一颗 4.7kΩ 普线上拉电阻。

- 加热：ESP32 的一个数字 GPIO 引脚接 MOS管模块的信号端（SIG）；MOS管的输入接 12V 电源，输出端连接加热块的两根加热线。

C. 力学与运动控制回路

- 称重读取：HX711 模块有 4 根信号线：VCC（接5V）、GND、DT和SCK（分别接 ESP32 的两个 GPIO 引脚，如 16 和 17）。传感器本身的红白黑绿四色线对应接在 HX711 输入端。

- 电机控制：步进电机驱动器（如 A4988）接 ESP32 的 STEP（步进信号）和 DIR（方向信号）两个引脚。驱动器的 1A, 1B, 2A, 2B 四根线接步进电机的四根线。

- 显示屏：使用 I2C 接口的 OLED 屏幕，只需 4 根线：VCC(3.3V)、GND、SDA（接 GPIO21）和 SCL（接 GPIO22）。

🚦 机器是怎样工作的？（软件逻辑概览）

当你把电路接好、程序写入 ESP32 后，它的工作逻辑非常直观：

- 预热：ESP32 读取热敏电阻温度，通过 MOS管加热，将探针稳定在 70℃（PID控温）。

- 归位：电机驱动探针升至高处，显示屏提示 [Ready: Place Sample]。

- 下压检测：

  - 放置一块废料，按下启动按钮。

  - 电机转动，驱动探针以恒定速度（如 1mm/s）向下移动。

  - 探针接触废料并深入，ESP32 实时读取 HX711 传回的阻力值。

- 分类判定：

  - 探针深入了 1mm 时：

    - 压力 $< 300\,\text{g}$ $\rightarrow$ 塑料在 70℃ 已软化，判定为 PLA。

    - 压力 $> 1500\,\text{g}$ $\rightarrow$ 塑料依然坚硬，判定为 ABS / PETG / 其他。

- 抬起：电机反转，将探针拔出并复位，屏幕显示结果。